# Intro to SoCet I Final Report: 
Matrix Calculator

- Introduction

- Project Description:
This project aims to develop a matrix math coprocessor designed in SystemVerilog. The system is implemented on an virtual FPGA. The system process FPGA keypads, manages a small memory architecture (SRAM), executes mathematical operations (ALU), and drives the FPGA user display interface.

- Motivation:

- Applications:

- Technical Background

- Mathematical Theory for matrices:
Addition, Multiplication, Transposition

- Hardware concepts:
FSM, memory, interfaces, protocol

- Top-Level System Architecture

- System Overview:
The data pipeline of the system starts with the user input by pressing buttons on the FPGA keypad, the Main controller FSM controls the input process and stores each user input into the memory, then the Main controller FSM triggers the arithmetic logic units and the ALUs returns the calculated back to main controller and writes to the memory, then the display controller is triggered and reads the results in memory and renders the display on the FPGA user interface.

- Top level RTL Diagram

- Submodule FSM and RTL Design

- Main Controller FSM

- Matrix and Memory Interfaces Communication Design RTL Diagram

- Matrix Interface RTL Diagram

- Memory Interface Design

- Memory Module RTL Diagram

- RTL Implementation Details

- Memory Allocation:
Uses a 512 bit memory with base address of 2 matrices and results (BASE_M1 = 0x000, BASE_M2 = 0x080, BASE_OUT = 0x100) to prevent data corruption.

- Memory pointers in main controller:

- Implementation details:
Transpose, Addition, and Multiplication Module

- Verification

- Independent Module Testbenches:
Test benches are included for each individual modules like main, add, vector, transpose, and display modules to verify that the logic is correct and that each module performed correct calculations.

- Main Controller Testbench:
The main controller testbench consist of virtual keypad presses sent from tb_sm_main to the design under test. The verification method is via checking if the output of the entire matrix to the terminal is correct.

- Visual verification:
For the main controller testbench, we use the Questa Sim waveform checker to confirm that the states of the finite state machine is correct.

- Project Result Demo and Operation Procedure

- A guide on FPGA simulation of the calculator

Suppose we want to perform the following matrix operation on the matrix calculator:

The simulation starts with prompting user to input the number of rows and columns to indicate the matrix size. The row and column size is limit to 9 by 9 at maximum. User press the row/column number first than press Y to confirm.

After the matrix size is set, then the simulation presumes with prompting user to input specific elements at a row m column n (r1c1). For example, the left figure shows user entering 123 in the first row second column, the right figure shows user entering 0 in the second row second column. The left and right light arrays shows the current position in the matrix. The entering procedure starts with pressing the number and press Z to shift to the left, and press Y to confirm. The maximum input number is limited to 3 digits because of the 8 digit display limit.

After all elements of the arrays are successfully input, the simulation let’s user to select the operation mode: addition (left, press A), multiplication (mid, press B), and transposition (right, press C); and then press Y to confirm. After mode is selected, the user interface would depend on the mode.

Addition mode: user is brought back to enter the second matrix elements, just enter in the same way as before.

Multiplication mode: user is brought back to enter the column number and then enter the matrix elements, just enter in the same way as before.

Transposition: user is brought to the calculation phase.

After sufficient elements are entered for the second matrix, it’s brought to the calculation phase. The calculation phase shows a fast blinking green light with some animations and turns blue when calculation is finished and is ready to view. The result can either be viewed in the terminal as full result. Or the user can use WABD as up left down right respectively, to move around the matrix elements and view the result. The left and right light arrays shows the current row and column of the element. In this case, the results shows that the element at the first row, first column is 2; at the first row, second column is 124.

- Discussions: Design Challenges and Solutions

- Challenge 1: Icarus library and Yosys Synthesizer
Problem: 
The ECE270 FPGA Yosys synthesizer optimizes the logic gates and results in a ($macc_v2) in the synthesized netlist sometimes when there’s a multiply and addition operator together. However, the Icarus library doesn’t contain the $macc_v2 logic gate and resulted in an error in the code and interrupts the execution.
Solution: 
The solution implemented is to use bitwise operations and case statement in the function mult_by_dim to replace the standard * operator, which prevents the $macc_v2 from being synthesized.

- Challenge 2: Handshake deadlock
Problem:
The matrix calculator was stuck at transition to select operation mode upon first testing. It turns out that the calculation finite state machines is waiting for the Main FSM valid signal to be high. However, the valid signal stayed high for more then two execution cycles for calculation module, which resulted it to fail to see the valid signal properly and resulted in a handshake deadlock.
Solution:
A 4 phase handshake was implemented that the main FSM module waits to acknowledge ready is high than drops valid to low before proceeding to the next state. This ensure the main FSM get’s done signal and proceed to further states.

- Challenge 3: Volatile Memory Read Error
Problem:
The output data of the final display state stayed at 00000000 upon testing on the FPGA simulator. The problem is that the memory drops the output data one cycle after the read enable pin drops low, this causes the calculation module to read the wrong signals (turned out to be 0) instead of the correctly calculated data.
Solution:
We implemented a temporary variable temp_rdata that acts like a register inside the main controller so that the memory reads the correct signal after read enable drops from high to low.

- Conclusion and Future Work

- Summary

- Future improvements

- References